Smart Facial Recognition Locking System

Student Name: John Joey Wright
Student ID: 20105823

1. Introduction

This project implements a smart access system using facial recognition, built around a Raspberry Pi. When a user presses a button, the system checks the ambient light level using a Grove Light Sensor. The current light condition and system status are displayed on a 16x2 LCD screen.

f light conditions are acceptable, a picture is taken using a connected camera (either a Raspberry Pi Camera Module or USB webcam). The image is uploaded to a local web server (running in a container on the student's laptop) which performs facial recognition using DeepFace, an open-source Python facial recognition library. If the face is verified as a known user, the system plays a happy beep and unlocks the door via a relay. If the face is not recognized, a sad beep is played and access is denied.

2. System Architecture Diagram

Figure 1: System Architecture showing hardware and logic flow between Raspberry Pi, camera, sensors, LCD, and the local web server.

3. Code Snippets and Explanations

Light Sensor Setup:

light_sensor = GroveLightSensor(channel=2)

if light_sensor.light < LIGHT_THRESHOLD:

lcd_update_bottom("Turn on light")

LCD Update Functions:

def lcd_update_top(line=""):

lcd.setCursor(0, 0)

lcd.write(line[:16].ljust(16))

def lcd_update_bottom(line=""):

lcd.setCursor(1, 0)

lcd.write(line[:16].ljust(16))

Camera Capture with OpenCV:

cap = cv2.VideoCapture(0)

ret, frame = cap.read()

cv2.imwrite("image1.jpg", frame)

cap.release()

Sending to Server:

files = {'image1': open('image1.jpg', 'rb')}

response = requests.post(SERVER_URL, files=files)

Decision Handling:

if verified:

happy_beep()

unlock_door()

else:

sad_beep()

4. Screenshots

Figure 1 idle

Figure 2 taking picture/uploading picture

Figure 3 User Verified

Figure 4 Light Level Low

5. Challenges Faced and Solutions

1. Raspberry Pi Camera Failure

- Problem: No data received from sensor error

- Solution: Switched to USB webcam and used OpenCV

2. LCD Display Issues

- Problem: Text overlapping or not clearing

- Solution: Created separate update functions for each line

3. Verification Feedback Delay

- Problem: LCD stuck on user name

- Solution: Reset LCD message after a delay using time.sleep(5)

4. Azure Issues:

- Problem: Azure student account has very strict limits and wont allow you to do a lot of things like make a container, use the facial recognition or create a logic app.

- Solution: I found no solution so I didn’t use Azure

5. Facial Recognition Service Selection

- Problem: Azure Cognitive Services were restricted by student subscription limits.

- Solution: Switched to using DeepFace, a powerful and easy-to-integrate open-source facial recognition library that runs entirely on the local machine.